A Seminar and technical writing report on

An AR/VR and AI-Guided Manual Assembly

Solution for Indian Manufacturing

Submitted in partial fulfilment of the requirements of the degree of

Master of Technology by

«Your Name»

Roll No. «24MEM3Rxx»

[Insert NIT Warangal emblem here]

Department of Mechanical Engineering

NATIONAL INSTITUTE OF TECHNOLOGY WARANGAL

2025-26

Abstract

Indian manufacturing still assembles a large share of its products by hand, from automotive wire harnesses and switchgear to pumps and electronic sub-assemblies. Manual assembly brings recurring problems: wrong or missed parts, incorrect sequence, slow onboarding of new and contract workers, out-of-date paper work instructions, and little traceability. This report proposes StepSure, a station-mounted AI assembly copilot that guides each manual step, verifies it with a camera and connected tools before the next step unlocks, trains new workers in virtual reality from the same digital process recipe, and is designed to add human-cobot collaboration later. The concept is informed by five real, independently verifiable patents covering projected AR guidance, AR glasses with assembly verification, machine-learning step recognition, VR assembly training, and AR/VR interaction with collaborative robots. Importantly, this report distinguishes a patent's technical disclosure from the scope of its granted claims, flags which records are granted patents and which are applications, and treats every financial figure as a clearly labelled planning assumption rather than a forecast. The report follows an academic-industrial structure covering the problem and opportunity, a patent portfolio overview, the unique value proposition, market analysis, business model, an intellectual-property and commercialization strategy, financial projections, and a 36-month implementation roadmap. Under the stated assumptions, the concept is technically feasible today and could reduce assembly errors and onboarding time materially, but it is not profitable in year one and reaches break-even only after roughly twelve to thirteen customers; both the willingness to pay and real-line computer-vision accuracy must be validated through a paid pilot before any wider commitment.

Keywords—Augmented reality, virtual reality, guided manual assembly, computer vision, pose estimation, assembly verification, human-robot collaboration, cobots, digital twin, freedom to operate, Indian manufacturing, Industry 4.0.

Contents

Abstract	ii

Chapter 1  Introduction	1

Chapter 2  Problem and Opportunity	3

Chapter 3  Patent Portfolio Overview	5

Chapter 4  Unique Value Proposition (UVP)	9

Chapter 5  Market Analysis	11

Chapter 6  Business Model	13

Chapter 7  IP and Commercialization Strategy	15

Chapter 8  Financial Projections	17

Chapter 9  Implementation Roadmap	19

Chapter 10  Conclusion	21

References	22

Chapter 1

Introduction

India's manufacturing sector is growing quickly and is central to national programmes such as Make in India and the production-linked incentive (PLI) schemes. Yet a large share of actual assembly work is still done by hand. Automotive component suppliers, electrical and electronics manufacturers, and general engineering plants assemble wire harnesses, switches, lamps, pumps, seats, meters and sub-assemblies manually, often across many low-volume product variants. As product mixes widen and quality expectations from original equipment manufacturers (OEMs) rise, manual reliance produces bottlenecks and quality escapes, creating an urgent need for affordable, worker-friendly digital assistance on the shop floor.

The common problems in manual assembly are well known: the wrong part is picked, a step is missed, the sequence is wrong, or a fastener is left loose; new and contract workers take days or weeks of buddy training to reach line speed; paper and PDF work instructions go out of date and are rarely read during work; and there is no reliable record of who did which step and when. Attrition on Indian shop floors makes this worse, because the training burden never ends.

Augmented reality (AR), virtual reality (VR) and computer vision (CV) offer a practical answer. AR shows the next instruction on top of the real work, by projection, on a screen, or in glasses. VR lets a trainee practise the whole assembly in a 3D virtual workplace before touching real parts. Computer vision and artificial intelligence (AI) let a camera recognise parts, hands and the current assembly state, so the system can confirm that a step was done correctly or warn the worker. A digital twin keeps a live digital copy of the station so the same process data can drive training, guidance and analytics. Later, the same platform can coordinate a collaborative robot (cobot) working next to the operator.

The evidence that AR guidance helps is long-standing. In a controlled experiment, Tang et al. (CHI 2003) found that overlaying 3D instructions on the actual work pieces reduced assembly errors by 82 percent compared with a paper manual, a computer screen and an unregistered head-mounted display [9]. This was a laboratory study with simple tasks, so real-factory gains will be smaller and more variable, but the direction is clear.

The technical foundation of the proposed solution is informed by five real patents, each covering a different part of the problem:

Projected-light guided assembly [1]: instructions projected directly onto the work.

AR glasses for assembly assistance and monitoring [2]: wearable guidance with camera-based verification.

Machine-learning assembly step recognition [3]: AI that recognises the current step and overlays a position cue.

Virtual reality assembly training [4]: step-sequenced VR training with haptic feedback.

AR/VR interaction with collaborative robots [5]: showing a robot's planned motion and aiding programming.

This report is explicitly patent-informed, not patent-protected. It uses the published technical disclosures of these patents to shape an architecture, while keeping a clear separation between what a patent teaches (its disclosure, which is public) and what it legally protects (its granted claims, which are narrower). It covers the problem and opportunity, a patent portfolio overview, the unique value proposition, market analysis, the business model, an IP and commercialization strategy, financial projections and an implementation roadmap, aiming at a realistic, affordable solution for Indian manufacturing.

Chapter 2

Problem and Opportunity

A. Existing Challenges in Manual Indian Assembly

Manual assembly lines in India face a set of connected operational and economic challenges that limit quality and productivity.

1) Quality escapes from human error:

A wrong part, a missed screw, a wrong connector or an incorrect torque passes down the line as rework or scrap, and sometimes reaches the OEM as a field complaint. For a Tier-1 or Tier-2 supplier, defective-parts-per-million (PPM) penalties and lost OEM ratings are a direct cost.

2) Slow onboarding and high attrition:

New and contract workers typically need days to weeks of buddy training to reach standard cycle time. Industry reporting places shop-floor attrition in auto components above 18-20 percent, so the training burden is continuous [11]. About 80 percent of Indian employers report difficulty finding skilled staff [10].

3) Out-of-date paper instructions and high variant count:

Paper and PDF standard operating procedures (SOPs) are hard to keep current, and are rarely consulted during the work itself. Indian suppliers run many low-volume variants, so the right instruction for the exact variant is often unclear at the station.

4) No verification and no traceability:

Most stations cannot confirm that each step was done correctly, and keep no per-unit record of who did which step and when. Vision inspection, where present, usually checks the finished product rather than guiding and checking each step in sequence.

5) The headset and platform risk:

Headset-first AR has struggled on Indian shop floors because of cost, comfort over a full shift, hygiene when shared across shifts, and narrow field of view. The risk is concrete: Microsoft stopped producing HoloLens 2 in 2024, and its industrial AR applications, Dynamics 365 Guides and Remote Assist, stop being available after 31 December 2026 [8]. Plants that invested in that platform must migrate.

B. Opportunity for an India-Specific Assembly Assistant

These constraints create a large opportunity for an affordable, headset-free, check-every-step assistant tuned to Indian conditions. The underlying demand is substantial:

Large, growing base: Indian auto-component turnover reached ₹6.73 lakh crore (about US$80.2 billion) in FY25, up 9.6 percent, with a 14 percent five-year CAGR (ACMA) [6].

Electronics scale-up: Indian electronics production was about ₹11.3 lakh crore in FY25, with mobile phones alone about ₹5.45 lakh crore (Economic Survey, as reported) [7].

Proven effect: AR guidance cut assembly errors by 82 percent in a controlled study [9].

A platform gap to fill: The Dynamics 365 Guides retirement leaves customers looking for a replacement [8].

The opportunity is strongly aligned with Make in India, the push toward Industry 4.0, and the Startup India Seed Fund Scheme, which offers up to ₹20 lakh for proof-of-concept and prototype development through approved incubators [12].

Chapter 3

Patent Portfolio Overview

The proposed approach is patent-informed. The design draws on the technical disclosures of five real patents to shape its architecture, while taking care to avoid infringement through amended mechanisms, different control logic and India-specific design choices, confirmed by a professional freedom-to-operate (FTO) review before any build or sale.

A note on kind codes, because it matters for this report's rigor. In the United States, a code ending in B1 or B2 means a granted patent; A1 means a published application that is not yet granted. The legal status of each record below (in force, lapsed or expired) was not verified on an official register for this report and is marked accordingly; these should be confirmed on the USPTO Patent Center, Espacenet and the Indian Patent Office InPASS search before the final submission.

1) Patent 1 – Light Guided Assembly System [1]:

Record: US 7,515,981 B2 (granted). Inventors: Paul Ryznar and James Ryznar. Owner: LightGuide, Inc. (formerly OPS Solutions LLC). Priority 2005; US term of about 20 years from filing points to roughly 2026, so it is near expiry if maintenance fees were paid; the later family members US 9,658,614 B2 and US 10,528,036, and EP 2673592 B2, run later. Status not officially verified.

This patent projects visual indicators (arrows, highlighted bins, locations, text) directly onto the work area and steps the operator through the assembly in order.

Contribution to the proposed system: it is the headset-free, station-mounted form of AR that suits a fixed overhead camera plus projector or screen. It is the primary prior art any projected-AR product must be compared against. Expiry of the 2005 patent does not make the idea free to use, because the later family patents remain in force and have been litigated.

2) Patent 2 – AR Assembly Assistance and Monitoring [2]:

Record: US 10,366,521 B1 (granted). Inventors: Jason L. Peacock, Joshua J. Watson, Scott M. Wilcox. Owner: Amazon Technologies, Inc. Filed about 2017; expected term to about 2037 if fees are paid. Status not officially verified.

Workers wear AR glasses that show the information for the current step. Camera image data is used to verify the assembly step, and the disclosure also shows safe walking paths and restricted areas in the glasses.

Contribution: it demonstrates that guidance and camera-based verification belong together, and adds safety-zone overlays that later connect naturally to cobot cells. It is important prior art showing the general idea of AR guidance plus verification is already known, so novelty must lie in how verification is done, not that it is done.

3) Patent 3 – Assembly Step Recognition Using AR [3]:

Record: US 11,715,300 B1 (granted). Inventors: Yuliang Guo, Xinyu Huang, Liu Ren (assignee probably Robert Bosch GmbH; confirm on the front page). Filed 2022; expected term to about 2042. Status not officially verified.

A first machine-learning model, trained on data generated from 2D images and matching 3D CAD models, turns a camera image into feature vectors; these are matched to feature-vector templates to identify the base and incoming components and the current step; a second model verifies that decision; and an AR position indicator shows where the part goes.

Contribution: this is almost exactly the AI object-recognition core of the proposed system, and therefore its single biggest FTO risk, especially for any sale into the United States. The proposed system must use a different recognition method (for example hand-action recognition or depth-based state comparison) or secure a licence, confirmed by counsel.

4) Patent 4 – Virtual Reality Training Simulator [4]:

Record: US 12,592,164 B2 (granted). Owner: Permco, Inc. (inventors to be taken from the front page). Filed 2022; granted 2026; expected term to about 2042. Status not officially verified.

A VR headset shows a virtual workbench, tools and components; the system shows an ordered list of steps and highlights the tool and location for each; one embodiment uses a haptic glove to teach workers to feel surface defects such as burrs.

Contribution: it matches the VR-training half of the proposed system and shows that ordered-step VR assembly training was patented in the US as recently as 2026. The design reuses one process recipe for both VR pre-training and floor guidance, so content is authored once.

5) Patent 5 – VR/AR Interaction with Collaborative Robots [5]:

Record: US 11,529,737 B2 (granted). Inventor: Kristen M. Stone. Owner: Raytheon Company. European family member EP 4096874. Priority 2020; granted 2022; expected US term to about 2040. Status not officially verified.

A wearable AR/VR headset shows the cobot's planned movement path over the real or virtual cell before it moves, and a VR training simulation feeds information to the robot workstation to help program the path.

Contribution: it covers exactly the future human-cobot extension of the proposed system. Any robot-intent overlay or hand-over feature must be checked against its claims and its European family. Note that such an overlay is an information aid, not a safety function; safety remains with certified robot safety systems under ISO 10218 and ISO/TS 15066.

Chapter 4

Unique Value Proposition (UVP)

StepSure offers a differentiated solution for Indian manual assembly across five parameters. The working name should be cleared on the Indian trademark registry before use.

1) Checked steps, not just shown steps:

Unlike display-only guidance, every step is verified by a camera and connected tools (part present and correctly placed, fastener count, hand path to the correct bin, torque-tool OK signal) before the next step unlocks. This is the core value and the main point of difference from projection-only and PDF-on-screen systems.

2) Headset-free on the line:

The default is a fixed overhead camera with a projector or screen, so there is no headset fatigue, the station is shared across all shifts, it works with gloves, and there is no dependence on a single headset vendor, an important lesson from the HoloLens and Dynamics 365 Guides exit. VR is used for training only, where a headset makes sense.

3) One recipe, three outputs:

A single digital process recipe (steps, parts, tools, checks, CAD references) drives projected or screen guidance, computer-vision checking, and automatically generated VR training content. Later it also drives cobot hand-over steps. Customers author the process once instead of paying three times for the same knowledge.

4) Built for Indian conditions:

Icon-first user interface with Hindi, Tamil, Marathi and Kannada voice and text for low-literacy and multilingual shop floors; pricing per station in INR; hardware sold close to cost; and on-site support in the main auto clusters (Pune, Chennai-Hosur, NCR, Bengaluru, Aurangabad).

5) Cobot-ready:

The same recipe and the safety-zone concept extend to a future Cobot Bridge that shows a robot's next move on the station display and runs guided hand-over steps, positioning the product for the shift toward human-cobot collaboration without forcing it on day one.

In one line: every step checked, every new worker productive in days, without headsets, at an Indian price.

Chapter 5

Market Analysis

1) Market Overview:

Indian auto-component turnover was ₹6.73 lakh crore (about US$80.2 billion) in FY25, growing at a 14 percent five-year CAGR [6], and electronics production was about ₹11.3 lakh crore in FY25 [7]. Both sectors employ large and growing manual-assembly workforces, which is the base StepSure serves. The served market for a first product is narrower: an estimated 400 named accounts in four clusters that match the ideal customer profile.

2) Adoption Gap:

Digital work-instruction and verification adoption on Indian manual lines is low, and much of what exists is either expensive imported AR or training-only VR. The gap is not AR for assembly, which already exists, but an affordable, headset-free, check-every-step system reusing one recipe for training, guidance and future cobot work.

3) Competitive Landscape:

The market includes several alternatives, each with a different focus:

Paper / PDF SOPs plus buddy training (status quo): free and familiar, but no checking, no data, slow onboarding.

LightGuide (US): mature projected-AR guidance with a patent portfolio; India pricing and support unverified.

Retrocausal (US): strong AI action-recognition assembly copilot; guidance UX and VR not core; India presence unknown.

PTC Vuforia (Expert Capture, Studio): large enterprise AR authoring ecosystem; priced per user by quote.

Microsoft Dynamics 365 Guides: the former market reference, ending 31 December 2026, which is a migration opportunity [8].

AutoVRse (Bengaluru): Indian industrial VR training with 100+ enterprise clients (self-reported); training-centric, not live line checking [13].

In-house machine vision (point inspection): accurate for fixed checks, but checks the product, not the step sequence, and gives no guidance or training.

StepSure occupies a position none of these fully covers: verified step-by-step guidance plus VR training from one recipe, headset-free, priced in INR.

4) Customer Segments:

Tier-1 and Tier-2 auto-component suppliers (harness, switches, lamps, brake and steering sub-assemblies) – the pilot segment.

Electronics and EMS and electrical (box build, meters, switchgear, EV chargers, BMS packs).

OEM plants (two- and four-wheeler, commercial vehicles, tractors) for scale and traceability.

Industrial training centres (ITIs, NSDC partners, university labs) for VR training seats.

Chapter 6

Business Model

The revenue model is a hybrid designed to lower customer CapEx and build recurring revenue. All prices below are planning assumptions that must be validated with pilot data.

1) Per-station software subscription (main stream):

The core recurring revenue is a per-station subscription of about ₹20,000 per station per month (₹2.4 lakh per station per year), billed annually. One prevented defect escape or a few days of faster onboarding per month should justify it.

2) Hardware kit near cost:

Each station kit (camera, edge AI computer, display or projector, mount and I/O) is sold at about ₹1.6 lakh, a modest margin over an estimated ₹1.35 lakh landed cost. Hardware is priced close to cost to reduce the buying barrier.

3) Onboarding and recipe set-up:

A one-time fee of about ₹3 lakh per customer for the first product family covers process digitisation, recipe authoring, and WMS/ERP or MES integration.

4) VR training licence:

For ITIs and training departments, about ₹30,000 per headset per year, with headsets supplied at cost. One headset serves many trainees.

5) Cobot Bridge add-on (future):

A later add-on of about ₹5,000 per station per month for robot-intent display and guided hand-over, once FTO is cleared.

A paid pilot (about ₹7 lakh for two stations over twelve weeks) is the entry point; it screens out non-serious prospects and funds a reference case study. Goods and services tax (GST) on software and services is generally 18 percent; confirm rates with a chartered accountant.

Chapter 7

IP and Commercialization Strategy

The strategy leverages existing patented concepts while building defendable, India-specific IP, and it rests on a correct reading of what patents actually protect.

1) Disclosure versus claims, and non-infringement:

A patent's disclosure (its description and drawings) is public and may be learned from and cited; its granted claims alone define what the owner can stop others from doing, and infringement requires every element of at least one claim. Patents are also territorial: a US patent is enforceable only in the US and does not stop anyone in India unless an Indian family member exists. The proposed non-infringement measures therefore include proprietary mechanical and mounting designs, a different computer-vision recognition method from the template matching in [3], and India-specific gripper and lighting design. None of this can be assumed safe without a professional FTO opinion, which must be obtained before any build or sale, starting with an Indian InPASS search and then the US and EU if exporting.

2) Patentability of the firm's own IP in India:

Section 3(k) of the Patents Act excludes a computer programme per se, but the 2025 Computer-Related Inventions guidelines and Delhi High Court case law allow inventions that show a technical effect. Any StepSure filing should be drafted around a technical effect, such as improved detection accuracy under occlusion or the control of tools and robots, rather than a better information display alone.

3) Technology transfer and alliances:

OEM integrators and automation vendors: bundle StepSure with existing lines (conveyors, torque tools, MES).

Industry bodies and Industry 4.0 centres: ACMA, CII, IMTMA, and SAMARTH Udyog Bharat 4.0 centres for demos and MSME reach (advisory, not funding).

Government schemes: Startup India Seed Fund Scheme for proof-of-concept and prototype support [12].

4) Future indigenous patent filings:

A depth-plus-action verification method robust to hands blocking the view.

India-specific multilingual SKU and label recognition using computer vision.

A headset-free robot-intent overlay for cobot hand-over at a manual station.

Chapter 8

Financial Projections

The financial model focuses on scaling the high-margin per-station subscription. Every figure is a clearly labelled assumption, not a forecast, and can be reproduced from the accompanying model script. A standard customer is assumed to run five stations.

| Metric | Year 1 (Development & Pilot) | Year 2 (Early Scale-Up) | Year 3 (Expansion & Break-Even) |
| --- | --- | --- | --- |
| Projected revenue | ₹0.2–0.3 Cr (2 paid pilots) | ₹1.2–1.5 Cr (3–5 customers) | ₹3–4 Cr (10–15 customers) |
| Gross margin | ~45% (incl. hardware) | ~65% (subscription-led) | ~70% |
| Key driver | Paid pilots, validation | Tier-1/2 adoption, referrals | Multi-plant expansion |
| Break-even | N/A | N/A | ~12–13 customers (≈62 stations) |

Basis and assumptions (all in INR):

Subscription ₹20,000/station/month; a 5-station customer is ₹12 lakh of annual recurring revenue (ARR) and about ₹23 lakh in the first year including kit and onboarding.

Year-1 spend is about ₹1.33 crore (team, capex, marketing, legal/IP, 10% contingency); with about ₹20 lakh of pilot revenue, the year-1 funding need is about ₹1.1 crore.

Steady-state fixed cost about ₹1.2 crore per year (9–10 people); recurring gross profit per 5-station customer about ₹9.6 lakh; break-even ≈ 1.2 Cr ÷ 9.6 L ≈ 12.5 customers.

Sensitivity: if subscription falls to ₹15,000/station/month, break-even rises to about 17 customers; if the average customer runs 10 stations, it falls to about 6–7. Stations per customer is the most important variable.

Salary basis: computer-vision engineers in India average about ₹7–7.9 lakh per year [14] and Unity developers about ₹4.6 lakh [15]; the model budgets above median to hire people who can ship. The uncomfortable truth is that the business is not profitable in year one in any scenario; break-even arrives in year three in the base case.

Chapter 9

Implementation Roadmap

1) Phase 1 – Prototype Development (0–6 months):

Build one station: guidance plus three check types (part present, fastener count, bin pick) on a real sub-assembly.

Target ≥ 95 percent check accuracy and ≤ 300 ms per check.

Author the VR trainer from the same recipe; secure Startup India seed funding or an incubator.

2) Phase 2 – Paid Pilot (6–12 months):

Install two stations at one pilot plant on a real product family, after measuring a 4-week baseline.

Gather data on error reduction, onboarding time, cycle-time overhead and uptime against agreed KPIs.

Convert the pilot to an annual contract and publish a case study with permission.

3) Phase 3 – Product and Scale-Up (12–24 months):

Release v1.0 with self-service Recipe Studio, dashboard and MES export, plus multilingual voice.

Add torque-tool and scanner integrations; win the second and third customers; build an integrator channel.

4) Phase 4 – Cobot Bridge and Expansion (24–36 months):

Ship the Cobot Bridge (robot-intent overlay and hand-over) with one cobot brand, after FTO clearance.

Reach the break-even customer count; begin indigenous Indian patent filings; explore Southeast Asia and the Middle East.

Chapter 10

Conclusion

AR/VR and AI-guided manual assembly is a real, patented and actively enforced field, spanning projected guidance, AR glasses with verification, machine-learning step recognition, VR assembly training and AR cobot interaction, from a 2005 priority to a 2026 grant. The market gap is not AR for assembly, which already exists, but an affordable, headset-free, check-every-step system for Indian suppliers that reuses one process recipe for training, guidance and future cobot work. StepSure is a realistic way to test that gap: one paid pilot, per-station pricing, India first. The two decisive unknowns are the customers' willingness to pay and real-line computer-vision accuracy, both testable within six months for under ₹50 lakh. Before any build or sale, a professional freedom-to-operate opinion is necessary, at minimum on the step-recognition patent family [3] and the LightGuide continuation family, including a check for Indian family members. Handled with that discipline, the concept can improve the quality and competitiveness of Indian manual assembly while contributing to the nation's Industry 4.0 goals.

References

[1] US Patent 7,515,981 B2, “Light guided assembly system,” granted 2009. https://patents.google.com/patent/US7515981B2/en

[2] US Patent 10,366,521 B1, “Augmented reality assembly assistance and monitoring,” Amazon Technologies Inc., 2019. https://patents.google.com/patent/US10366521B1/en

[3] US Patent 11,715,300 B1, “Systems and methods for providing product assembly step recognition using augmented reality,” 2023. https://patents.google.com/patent/US11715300B1/en

[4] US Patent 12,592,164 B2, “Virtual reality training simulator,” Permco Inc., 2026. https://image-ppubs.uspto.gov/dirsearch-public/print/downloadPdf/12592164

[5] US Patent 11,529,737 B2, “System and method for using virtual/augmented reality for interaction with collaborative robots in manufacturing or industrial environment,” Raytheon Co., 2022. https://patents.google.com/patent/US11529737

[6] ACMA, “Industry Performance Review FY2024-25,” 2025. https://acma.in/uploads/press-release/Press%20Release%20FY25.pdf

[7] Government of India, Economic Survey (electronics production FY25), as reported, 2025.

[8] Microsoft, “Dynamics 365 Guides and Remote Assist retirement (after 31 December 2026),” Microsoft Learn, 2025. https://learn.microsoft.com/en-us/dynamics365/mixed-reality/guides/admin-apps-solution-compatibility

[9] A. Tang, C. Owen, F. Biocca, W. Mou, “Comparative effectiveness of augmented reality in object assembly,” Proc. CHI 2003, pp. 73-80.

[10] ManpowerGroup, “2025 Global Talent Shortage Survey (India),” 2025.

[11] Autocar Professional, “India’s auto component industry runs into a deeper talent problem,” 2025.

[12] Government of India, DPIIT, “Startup India Seed Fund Scheme (SISFS),” 2021. https://www.indiascienceandtechnology.gov.in/funding-opportunities/startups/startup-india-seed-fund-scheme-sisfs

[13] AutoVRse, company and client information, 2025. https://evreporter.com/autovrse-raises-2-4-million-to-expand-ai-and-vr-training-for-automotive-manufacturing/

[14] AmbitionBox / Payscale, “Computer Vision Engineer salary, India,” 2025-26.

[15] Glassdoor, “Unity Developer salary, India,” 2026.